Tarunrajes

251901174

| Ex.No.2 | Study of Network cables. Understand different types of Network cables. Make a cross-wired cable and straight through cable using clamping/crimping tool. |
| --- | --- |
| Date: |  |

- Understand different types of network cable.

Different type of cables used in networking are:

  - Unshielded Twisted Pair (UTP) Cable

  - Shielded Twisted Pair (STP) Cable

  - Coaxial Cable

  - Fibre Optic Cable

| Cable type | Category | Maximum Data Transmission | Advantages/ Disadvantages | Application/Use | Image |
| --- | --- | --- | --- | --- | --- |
| UTP | Category 3 | 10 bps | Advantages - Cheaper in cost - Easy to install as they have a smaller overall diameter. Disadvantages - More prone to (EMI) Electromagnetic interference and noise | 10Base-T Ethernet |  |
|  | Category 5 | Up to 100 Mbps |  | Fast Ethernet, Gigabit Ethernet |  |
|  | Category 5e | 1Gbps |  | Fast Ethernet, Gigabit Ethernet |  |
| STP | Category6,6a | 10Gbps | Advantages - Shielded. - Faster than UTP. - Less susceptible to noise and interference Disadvantages - Expensive - Greater installation effort | Gigabit Ethernet, 10G Ethernet (55m) Widely used in data centres |  |
| SSTP | Category 7 | 10Gbps |  | Gigabit Ethernet, 10G Ethernet (100m) |  |

| Coaxial cable | RG-6 RG-59 RG-11 | 10-100Mbps | - High bandwidth - Immune to interference - Low loss bandwidth - Versatile - Disadvantages - Limited distance - Cost - Size is bulky | Speed of signal is 500m Television network High speed internet connections |  |
| --- | --- | --- | --- | --- | --- |
| fibre optics cable | Single mode Multi mode | 100Gbps | Advantages - High speed - High bandwidth - High security - Long distance Disadvantages - Expensive - Requires skilled installers | - Maximum distance of fibre optics cable is around 100meters |  |

- Make Your Own Ethernet Cross-Over Cable/ Straight cable

Tools and parts needed:

- Ethernet cabling. CAT5e is certified for gigabit support, but CAT5 cabling works as well, just over shorter distances.

- A crimping tool. This is an all-in-one networking tool shaped to push down the pins in the plug and strip and cut the shielding off the cables.

- Two RJ45 plugs.

- Optional two plug shields.

  Ethernet Cable (CAT5e/CAT5)

- CAT5e is recommended because it supports Gigabit Ethernet (1000 Mbps).

- CAT5 also works but supports lower speeds and shorter distances.

  Crimping Tool

- A networking tool used to:

  - Strip the outer insulation of the cable.

  - Arrange and trim the wires.

  - Crimp (press) the RJ45 connector onto the cable.

  RJ45 Connectors (2)

- Plastic connectors fitted at both ends of the Ethernet cable.

- Used to connect the cable to computers, switches, routers, etc.

  Plug Shields (Optional)

- Protective covers placed over the RJ45 connectors.

- Prevent damage to the locking clip and reduce strain on the cable.

RESULT: